Ce dossier technique comporte 25 pages, numérotées de 1 / 25 à 25 / 25

Assurez-vous que cet exemplaire est complet.

S’il est incomplet, demandez un autre exemplaire au chef de salle.

DOSSIER TECHNIQUE

BaccalaurÉat Professionnel

AÉRONAUTIQUE

OPTION : AVIONIQUE

ÉPREUVE E2 – EXPLOITATION DE LA DOCUMENTATION TECHNIQUE

SOMMAIRE

| PAGE 2 | PRÉSENTATION DU RADAR MÉTÉOROLOGIQUE |
| --- | --- |
| ANNEXE 1 / Page 3 | SERVICE BULLETIN n° 2018-01-28-AB337-400-149 |
| ANNEXE 2 / Page 4 | ILLUSTRATED PART CATALOG |
| ANNEXE 3 / Page 5 | PRÉSENTATION DE L’AVION AirBo 377-400 : Constitution |
| ANNEXE 4 / Page 6 | ZONING |
| ANNEXE 5 / Page 7 | VUE DE LA TRANSMISSION MÉCANIQUE DE L’ANTENNE DU RADAR MÉTÉOROLOGIQUE CARACTÉRISTIQUES DES ÉLINGUES / FORMULAIRE |
| ANNEXE 6 / Page 8 | TASK 53-15-11-000-801 RADOME - REMOVAL/INSTALLATION |
| ANNEXE 7 / Page 12 | 34-41-11 PB 401 CONF 00 ANTENNA ASSEMBLY- WEATHER RADAR -REMOVAL/INSTALLATION |
| ANNEXE 8 / Page 16 | TASK 24-41-00-861-801-A ENERGIZE THE AIRCRAFT ELECTRICAL CIRCUITS FROM THE EXTERNAL POWER A |
| ANNEXE 9 / Page 17 | GÉNÉRATION ÉLECTRIQUE – ALIMENTATION DE L’E/R RADAR MÉTÉO WRX1 |
| ANNEXE 10 / Page 18 | TASK 34-41-00-710-802-A OPERATIONAL TEST OF THE WEATHER-RADAR ANTENNA SCANNING |
| ANNEXE 11 / Page 19 | WIRING DIAGRAM DE L’INSTALLATION RADAR MÉTÉOROLOGIQUE |
| ANNEXE 12 / Page 20 | SYNOPTIQUE DE L’INSTALLATION RADAR MÉTÉOROLOGIQUE |
| ANNEXE 13 / Page 21 | AWM NAVIGATION WEATHER RADAR SYS 1 - ANTENNA |
| ANNEXE 14 / Page 23 | DIGITAL INTERFACE SPECIAL FOR WXR/PWS |
| ANNEXE 15 / Page 24 | OSCILLOGRAMMES ARINC |
| ANNEXE 16 / Page 25 | TROUBLE SHOOTING – TASK 34-41-00-810-803 FAILURE OF THE WEATHER RADAR ANTENNA |

PRÉSENTATION DU RADAR MÉTÉOROLOGIQUE

Dès les premières années de la Seconde Guerre mondiale, le développement des magnétrons de puissance a permis de monter des radars à bord des avions.

Avec l’application des radars à la détection des précipitations, des radars météorologiques aéroportés ont rapidement vu le jour après le conflit. Ceux-ci sont maintenant largement répandus dans l’aviation commerciale et dans la plupart des avions d’affaire.

Le radar météo permet en outre d'assurer les vols en toute sécurité. Situé dans le nez, il balaye le ciel devant celui-ci. Ces radars sont basés sur l'effet Doppler et permettent de mesurer la taille des gouttes d'eau dans les nuages, renseignant ainsi les pilotes sur le type de précipitation.

Cela leur permet notamment d'éviter les orages, dangereux en aviation, non pas par la foudre, mais par les conditions givrantes et les turbulences que l'on est susceptible de rencontrer.

Les radars donnent une image des conditions atmosphériques vers lesquelles l’appareil se dirige et permettent ainsi au pilote d’éviter les zones dangereuses comme les orages. Leur portée maximale habituelle est de 180 NM mais ils peuvent être limités sur demande à des portées plus courtes, de 30 à 80 NM, pour améliorer la résolution et avoir ainsi plus de détails.

Principe de fonctionnement :

Le radar météorologique travaille dans la gamme SHF (Supra High Frequency) en bande C, S ou X suivant la technologie du radar.

Le radar météorologique émet un signal hyperfréquence sous la forme d’impulsions électromagnétiques. Ce signal est émis en direction de la cible. Une petite partie de l'énergie transmise est réfléchie par la cible dans la direction du radar. Cette énergie renvoyée par la cible jusqu'au radar est appelée ECHO.

La technologie utilisée pour fabriquer cette onde est la même que celle des fours micro-ondes qui travaillent également en SHF.

Si cette gamme de fréquence permet de détecter les molécules d’eau avec une très grande résolution, elle présente également l’inconvénient, d’être dangereuses pour l’être humain en cas d’exposition prolongée. C’est pourquoi des précautions particulières sont à prendre lorsque l’on effectue des essais, et en particulier le respect absolu des zones de sécurité.

ANNEXE 1

AB 377-400

SERVICE BULLETIN

SUMMARY

MANDATORY MANDATORY MANDATORY

ATA SYSTEM: 34

TITRE :	NAVIGATION – RADAR METEOROLOGIQUE

REMPLACEMENT DE L’ANTENNE RADAR METEO ET DE SON ELECTRONIQUE ASSOCIEE PAR L’ENSEMBLE OMESSON-THOMEGA OT-008A ou OT-008B

** CONF ALL

- MODIFICATIONS

| CLASSIFICATION MODIFICATION |  |
| --- | --- |
| MAJEUR | NONE |
| MINEUR | 2017-185-14 / 2017-06-06 |

- RAISON / DESCRIPTION / CONSEQUENCES OPERATIONNELLES :

Basé sur les performances des données du radar météorologique, en accord avec les analyses de l’EASA (European Aviation Safety Agency), celle-ci et la société AIRBO ont approuvé le lancement d’une campagne de retrofit en mesure préventive sur AB 377-400 actuellement équipés de l’Antenne Omesson-Thomega OT-007A ou OT-007B.

Ce Service Bulletin requiert l’installation d’une nouvelle antenne Radar Météo et de son électronique associée Omesson-Thomega Réf. OT-008A ou Réf. OT-008B suite à la modification 2017-185-14 du 06 juin 2017.

- DESCRIPTION

Pour accomplir ce Service Bulletin, il est nécessaire d’utiliser :

TASK 34-41-11-000-801 ANTENNA ASSEMBLY – WEATHER RADAR (7SQ - 11SQ)

DEPOSE / REPOSE DE L’ANTENNE RADAR METEOROLOGIQUE

- CONFORMITÉ

Classification MANDATORY (OBLIGATOIRE)

- MAIN D’OEUVRE

Temps estimé d’intervention basé sur le coût direct de la main-d’œuvre.

| TASK 34-41-11-000-801 ANTENNA ASSEMBLY – WXR (7SQ - 11SQ) Removal / Installation DESIGNATION |  |
| --- | --- |
| Ouverture des accès | 0.50 |
| SUR L’AVION |  |
| Dépose / Repose de l’antenne et de son électronique associée | 4.00 |
| Test du Radar Météorologique | 0.50 |
| Remise en conformité | 0.50 |
| TOTAL HEURES MAIN D’OEUVRE | 5.50 |

Date: 01 janv. 18				         Service Bulletin n° 2018-01-28-AB337-400-149

Revision: 00 / janv. 18							     Page: 1

SERVICE BULLETIN

- PROCÉDURE

TASK 34-41-11-000-801 Antenna assembly – Weather Radar (7SQ – 11SQ) Removal / Installation

Ouverture des accès :	Subtask 34-41-11-010-050

Subtask 34-41-11-010-051

| ZONE DE TRAVAIL ET PANNEAUX D’ACCES |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ZONE | ACCES ET LOCALISATION |  |  |  |
|  | FR1 | Accès |  | Radome 110 |  |
|  | FR7- FR10 | Accès |  | Porte 811 |  |
| RESSOURCES HUMAINES |  |  |  |  |  |
| Heures de main d’œuvre |  |  |  |  | 5.50 |
| Nombre de Personne minimum |  |  |  |  | 2 |
| Temps estimé de la sous-tâche |  |  |  |  | 2.75 |
| Compétence |  |  |  |  | B2 |
| Matériel nécessaire pour effectuer les travaux |  |  |  |  |  |
| NOUVEAU PART. NUMBER ZONE |  | Qté | DÉSIGNATION |  |  |
| OMTH-008A ou OMTH-008B |  | 1 | Weather Radar Antenna Assembly |  |  |

(a). Dépose (se référer à la TASK 34-41-11-000-801-A)

FIN 11SQ et 7SQ

1	Weather Radar antenna Assembly	OMTH007 A ou OMTH-007B

(b). Pose (se référer à la TASK 34-41-11-400-801-A)

FIN 11SQ et 7SQ

1	Weather Radar antenna Assembly	OMTH008 A ou OMTH-008B

- TEST

(1). Subtask 34-41-00-865-052A C/B Closing

Retirer le (s) clip (s) de sécurité et la (les) flamme (s) et fermer le (s) disjoncteur (s) comme spécifié dans la dépose/repose de l’ensemble antenne du radar météorologique (11SQ et 7SQ).

(2). Subtask 34-41-00-710-050-B Testing

Effectuer les tests après l’installation de l’ensemble antenne du radar météorologique (11SQ et 7SQ).

- FERMETURE

(a). Faire la procédure de fin comme spécifié dans l'installation de l'antenne radar météorologique.

(b). Remettre les systèmes et l'avion en état de fonctionnement normal

Date: 01 janv. 18				         Service Bulletin n° 2018-01-28-AB337-400-149

Revision: 00 / janv. 18							     Page: 2

ANNEXE 2

ANNEXE 3

Présentation de l’AirBo A377- 400 : CONSTITUTION

ANNEXE 4

CONVERSION de rad/s en tr /min :

en rad/s

N en tr/min

CALCUL DU POIDS :

P en N

m en kg

g = 9.81 m/s2

CALCUL DE RAPPORT DE RÉDUCTION :

r < 1 c’est une réduction : la vitesse de sortie est inférieure

à la vitesse d’entrée

r > 1 c’est une multiplication : la vitesse de sortie est supérieure

à la vitesse d’entrée

N sortie = fréquence de sortie en tr/min

N entrée  = fréquence d’entrée en tr/min

= vitesse de sortie en rad/s

= vitesse d’entrée en rad /s

CALCUL DU COUPLE :

∁=d×F

C en N.m

d en m

F en N

CALCUL DE LA PUISSANCE MÉCANIQUE DE ROTATION :

P en W

C  en N.m

en rad/s

CALCUL DU RENDEMENT :

P sortie en W

P entrée en W

 = rendement

ANNEXE 5

RÉFÉRENCES:

| Élingues |  |
| --- | --- |
| Référence | Charge maximale en N |
| T6321 | 60 |
| T6322 | 260 |
| T6323 | 460 |
| T6324 | 660 |
| T6325 | 860 |
| T6326 | 1060 |

RECOMMANDATIONS :

- Utiliser les dispositifs d’arrimage conformément à la notice d’utilisation remise avec le matériel.

- Ne pas utiliser le système d’arrimage pour élinguer des charges.

- Les sangles ne doivent pas être nouées.

- Ne pas utiliser les sangles d’arrimage sur des surfaces rugueuses ou au contact d’arêtes vives sans l’interposition d’accessoires de protection.

- Les dispositifs d’arrimage doivent être utilisés entre -40°C et 100°C.

- Pour une utilisation du système d’arrimage en présence de produits chimiques, consulter le fabricant.

- Adapter les pièces d’extrémités aux points d’accrochage conformément aux prescriptions du fabricant.

- Ne pas utiliser de pièces autres que celles préconisées par le fabricant pour manœuvrer les tendeurs.

- Les systèmes d’arrimage doivent être stockées dans un endroit frais et sec, et doivent être protégés de la lumière et de tout risque d’endommagement mécanique.

Produit du nombre de dents des roues menantes

Produit du nombre de dents des roues menées

Secteur denté Z2

Pignon moteur Z1

ANNEXE 6 (1/4)

TASK 53-15-11-000-801   RADOME - REMOVAL/INSTALLATION

Removal of the Radome (110AL)

WARNING : 	MAKE SURE THAT THE CIRCUIT BREAKERS RELATED TO THE WEATHER

RADAR ARE OPENED, SAFETIED AND TAGGED.

1. Reason for the Job

Self-Explanatory

2. Job Set-up Information

A. Fixtures, Tools, Test and Support Equipment

--------------------------------------------------------------------------------------------------------------------------------------

REFERENCE 					QTY DESIGNATION

--------------------------------------------------------------------------------------------------------------------------------------

No specific 						circuit breaker(s) safety clip(s)

No specific 						warning notice

No specific 						access platform 2.5 m (8 ft. 2 in.)

No specific 						access platform 4 m (13 ft. 1 in.)

98D53103013001 					1 TROLLEY - RADOME

****		 					4 STRAPS

B. Referenced Information

--------------------------------------------------------------------------------------------------------------------------------------

REFERENCE 					QTY DESIGNATION

--------------------------------------------------------------------------------------------------------------------------------------

24-42-00-861-801 					Energize the Ground Service Network

53-15-11-010-801 					Opening of the Radome (110AL).

53-15-11-991-001-A 				Fig. 401

R 53-15-11-991-021 				Fig. 402

**ON A/C 001-099, 101-159, 201-299, 501-549, 551-552,

53-15-11-991-002-A 				Fig. 403

**ON A/C 160-199, 301-349, 553-599,

53-15-11-991-002-B 				Fig. 403A

**ON A/C ALL

3. Job Set-up

(Ref. Fig. 401/TASK 53-15-11-991-001-A)

Subtask 53-15-11-861-056

A. Energize the ground service network

(Ref. TASK 24-42-00-861-801).

Subtask 53-15-11-010-082

B. Get Access to the Avionics Compartment

(a) Put the access platform in position at the access door 811.

(b) Open the access door 811.

Subtask 53-15-11-865-050

C. Open, safety and tag this(these) circuit breaker(s):

-------------------------------------------------------------------------------------------------------------------------------------

PANEL 			DESIGNATION		FIN 			LOCATION

-------------------------------------------------------------------------------------------------------------------------------------

721VU 			WXR 1 			9SQ1 			K03

722VU 			WXR 2 			9SQ2 			E49

Subtask 53-15-11-941-050

D. Safety Precautions

(a) In the cockpit, put a warning notice in position on the center pedestal to tell persons not to operate the radar system.

ANNEXE 6 (2/4)

Subtask 53-15-11-010-050-A

E. Opening of the Radome

WARNING: 	IN THE COCKPIT, PUT A WARNING NOTICE IN POSITION ON THE CENTER

PEDESTAL TO TELL PERSONS NOT TO OPERATE THE RADAR SYSTEM.

CAUTION:	DO NOT USE THE LOCALIZER ANTENNA AT THE BOTTOM OF THE RADOME ASA SUPPORT.

IF YOU DO, YOU CAN CAUSE DAMAGE.

- Put the access platform in position at zone 110.

- Open the radome (Ref. TASK 53-15-11-010-801)

- Put the plastic bag on the radome.

Subtask 53-15-11-480-050-A

F. Installation of the Sling

(Ref. Fig. 401/TASK 53-15-11-991-001-A)

- Install the STRAP - RADOME (98F53101000000) or STRAP - R/I, RADOME (98A53601000000) or STRAP - RADOME (98F53103500000).

- Release the telescopic rods; let the radome move in the closed direction until the strap is tight.

- Adjust the radome-opening angle at 45⁰ approximately.

- Adjust the cable length with the bottle screw until you get an equal tension on both straps.

Installation of the Sling

Figure 401/TASK 53-15-11-991-001-A

ANNEXE 6 (3/4)

4 . Procedure

Subtask 53-15-11-020-050-A

A. Disconnection of the Telescopic Rods (1)

(Ref. Fig. 402/TASK 53-15-11-991-021)

CAUTION: 	MOVE THE TELESCOPIC ROD CAREFULLY TO PREVENT DAMAGE TO THE RADAR.

(1) For the telescopic rods (P/N 56501-1):

(a) Remove and discard the cotter pins (2).

(b) Remove the nuts (3), the washers (4).

(c) Remove the screws (5).

(2) For the telescopic rods (P/N ST351-1):

- Remove the pins (6).

- Disconnection of the Telescopic Rods

- Figure 402/TASK 53-15-11-991-021

ANNEXE 6 (4/4)

4 . Procedure (Suite)

Subtask 53-15-11-020-051

B. Removal of the Radome

CAUTION: MOVE THE RADOME CAREFULLY TO PREVENT DAMAGE TO THE RADAR.

(Ref. Fig. 403/TASK 53-15-11-991-002-A)

- Remove the nuts (5).

- Remove the screws (6).

- Remove the bonding strips.

- Remove and discard the cotter pins (1).

- Remove the nuts (2) and washers (3).

- Remove the upper screws (4).

- Remove the radome and install it in TROLLEY – RADOME (98D53103013001).

- Remove the straps from the radome.

Adjustment of the Radome Opening Handle

R Figure 403/TASK 53-15-11-991-002-A

ANNEXE 7 (1/4)

34-41-11 PB 401 CONF 00

ANTENNA ASSEMBLY- WEATHER RADAR -REMOVAL/INSTALLATION

TASK 34-41-11-000-801-A

Removal of the Weather Radar Antenna Assembly

FIN: 7SQ , 11SQ

1.    Reason for the Job

Self explanatory

2     Job Set-up Information

A.   Fixtures, Tools, Test and Support Equipment

| REFERENCE | QTY | DESIGNATION |
| --- | --- | --- |
| No specific | AR | ACCESS PLATFORM 3M (10 FT)- ADJUSTABLE |
| No specific | AR | ACCESS PLATFORM 5M (16 FT)- ADJUSTABLE |
| No specific | AR | CAP - BLANKING |
| No specific | AR | SAFETY CLIP - CIRCUIT BREAKER |

B.   Work Zones and Access Panels

| ZONE/ACCESS | ZONE DESCRIPTION |
| --- | --- |
| 110 | RADOME |
| 110AL, 811 |  |

C.   Referenced Information

| REFERENCE | DESIGNATION |
| --- | --- |
| (Ref. 24-42-00-861-801-A). | Energize the Ground Service Network |
| (Ref. 53-15-11-010-801-A). | Opening of the Radome |
| Weather Radar Antenna Assembly SHEET 1 |  |

**ON A/C 001-099, 101-150, 201-299, 301-349,

34-41-11-991-001 Fig. 401

**ON A/C 151-199, 501-549, 551-599,

34-41-11-991-001-A Fig. 401A

**ON A/C ALL

3.    Job Set-up

Subtask 34-41-11-861-050-A

A.   Energize the ground service network

(Ref. AMM TASK 24-42-00-861-801)

Subtask 34-41-11-010-050-A

B.   Get access to the avionics compartment.

(a)  Put the ACCESS PLATFORM 3M (10 FT) ADJUSTABLE in position at the access door 811.

(b)  Open the access door 811.

Subtask 34-41-11-865-050-A

C.   Open, safety and tag this (these) circuit breaker(s):

| PANEL | DESIGNATION | FIN | LOCATION |
| --- | --- | --- | --- |
| 721VU | WXR 1 | 9SQ1 | K03 |
| 722VU | WXR 2 | 9SQ2 | E49 |

Subtask 34-41-11-010-051-A

WARNING:    DO NOT OPEN THE RADOME IF THE WIND SPEED IS MORE THAN 35 KNOTS.

IF THE RADOME IS OPEN IN WINDS HIGHER THAN 35 KNOTS IT CAN BREAK FROM ITS HINGES AND CAUSE INJURY.

D.   Get Access

NOTE: Two persons are necessary to open the radome and to remove the antenna.

(a)  Put the ACCESS PLATFORM 5M (16 FT) ADJUSTABLE in position under the radome in the zone 110.

(b)  Open the radome 110AL (Ref. AMM TASK 53-15-11-010-801)

WARNING:     BE CAREFUL WHEN YOU REMOVE OR INSTALL THE RADAR ANTENNA ASSEMBLY TO PREVENT INJURY TO PERSONS AND/OR DAMAGE.

THE RADAR ANTENNA ASSEMBLY (ANTENNA/DRIVE) IS HEAVY.

CAUTION:      DO NOT USE THE LOCALIZER ANTENNA AT THE BOTTOM OF THE RADOME AS A SUP- PORT.

IF YOU DO YOU CAN CAUSE DAMAGE.

ANNEXE 7 (2/4)

4.    Procedure

(Ref. Fig. Weather Radar Antenna Assembly SHEET 1)

R **ON A/C 001-099, 101-150, 201-299, 301-349,

Subtask 34-41-11-020-050-A

A.   Removal of the Antenna

(a)  Remove the four screws (18) and the lock washers (19) which connect the flange of the waveguide (14) to the antenna (1).

NOTE: During the removal of the antenna:

-  one person must hold the antenna

-  one person must hold the position indicator (13) to prevent damage to the drive caused by fast movement of the counterbalance spring.

(b)  Remove the eight screws (17) and the lock washers (16) which connect the ring mount (15) to the antenna (1) and remove the antenna.

(c)  Slowly release the position indicator (13).

(d)  Put the antenna (1) in a plastic bag and a correct container.

Subtask 34-41-11-020-051-A

B.   Removal of the Antenna Drive

(a)  Disconnect the electrical connectors (12).

(b)  Loosen the quick disconnect clamp (5) and disconnect the waveguide (4) from the antenna drive (2).

(c)   Remove and discard the O-ring (3).

(d)  Put CAP - BLANKING on the disconnected electrical connectors and on the flange of the waveguide (4).

(e)  Remove the two lower screws (11) and the washers (10).

(f)   Loosen the two upper screws (6). Do not remove them.

(g)  Lift the antenna drive (2) until the holes (8) are at the same level as the two upper screws (6).

(h)  Remove the antenna drive (2) and put it in a plastic bag and a correct container.

Figure 34-41-11-991-00100-00-A / SHEET 1/1 - Weather Radar Antenna Assembly

** ON A/C ALL

ANNEXE 7 (3/4)

TASK 34-41-11-400-801-A

Installation of the Weather Radar Antenna Assembly

FIN : 7SQ , 11SQ

1.    Reason for the Job

Self explanatory

2     Job Set-up Information

- Fixtures, Tools, Test and Support Equipment

| REFERENCE | QTY | DESIGNATION |
| --- | --- | --- |
| No specific | AR | ACCESS PLATFORM 3M (10 FT)- ADJUSTABLE |
| No specific | AR | ACCESS PLATFORM 5M (16 FT)- ADJUSTABLE |
| No specific |  | Torque wrench: range to between 0.23 and 0.26 m.daN (20.35 and 23.01 lbf.in) |
| No specific |  | Torque wrench: range to between 2.69 and 3.22 m.daN (19.84 and 23.75 lbf.ft) |

B.   Consumable Materials

| REFERENCE | DESIGNATION |
| --- | --- |
| (Material No. 04TMB2) | Varnish-- Electrical Bonding Structure |

- Work Zones and Access Panels

| ZONE/ACCESS | ZONE DESCRIPTION |
| --- | --- |
| 110 | RADOME |
| 110AL, 811 |  |

- Expendable Parts

| FIG.ITEM | DESIGNATION | IPC-CSN |
| --- | --- | --- |
| 3 | O-ring | 34-41-41-01-040 |

- Referenced Information

| REFERENCE | DESIGNATION |
| --- | --- |
| (Ref. 20-28-00-912-810-A). | Electrical Bonding - General Maintenance Procedure |
| (Ref. 24-42-00-861-801-A). | Energize the Ground Service Network |
| (Ref. 24-42-00-862-801-A). | De-energize the Ground Service Network |
| (Ref. 34-41-00-710-802-A). | Operational Test of the Weather-Radar Antenna Scanning |
| (Ref. 53-15-11-410-801-A). | Closing of the Radome |
| Weather Radar Antenna Assembly SHEET 1 |  |

3.    Job Set-up

Subtask 34-41-11-860-050-A

A.   Aircraft Maintenance Configuration

- Make sure that the ground service network is energized

(Ref. AMM TASK 24-42-00-861-801).

(b)  Make sure that the ACCESS PLATFORM 3M (10 FT)- ADJUSTABLE is in position at the access door 811.

(c)  Make sure that the ACCESS PLATFORM 5M (16 FT)- ADJUSTABLE is in position under the radome in the zone 110.

WARNING:   DO NOT OPEN THE RADOME IF THE WIND SPEED IS MORE THAN 35 KNOTS.

IF THE RADOME IS OPEN IN WINDS HIGHER THAN 35 KNOTS IT CAN BREAK FROM ITS HINGES AND CAUSE INJURY.

(d)  Make sure that the access door 811 and the radome 110AL are open

Subtask 34-41-11-865-051-A

B.   Make sure that this (these) circuit breaker(s) is(are) open, safetied and tagged:

| PANEL | DESIGNATION | FIN | LOCATION |
| --- | --- | --- | --- |
| 721VU | WXR 1 | 9SQ1 | K03 |
| 722VU | WXR 2 | 9SQ2 | E49 |

WARNING: BE CAREFUL WHEN YOU REMOVE OR INSTALL THE RADAR ANTENNA ASSEMBLY TO PREVENT INJURY TO PERSONS AND/OR DAMAGE.

THE RADAR ANTENNA ASSEMBLY (ANTENNA/DRIVE) IS HEAVY.

CAUTION:     DO NOT USE THE LOCALIZER ANTENNA AT THE BOTTOM OF THE RADOME AS A SUPPORT. IF YOU DO YOU CAN CAUSE DAMAGE.

ANNEXE 7 (4/4)

4.    Procedure

(Ref. Fig. Weather Radar Antenna Assembly SHEET 1)

Subtask 34-41-11-420-050-A

- Installation of the Antenna Drive

WARNING:   OBEY THE MANUFACTURER'S INSTRUCTIONS WHEN YOU USE THIS/THESE MATERIAL/S.

USE PROTECTIVE CLOTHING, RUBBER GLOVES AND GOGGLES.

- Clean the component interfaces and the adjacent area.

- Do a visual inspection of the component interfaces and the adjacent area.

- Make sure that the two upper screws (6) are sufficiently loose.

- Lift the antenna drive (2) up to its support and engage it in the two upper screws (6) and washers (7).

- Install the two lower screws (11) and washers (10).

- Torque the four screws (6) and (11) to between 2.69 and 3.22 m.daN (19.84 and 23.75 lbf.ft).

- Make sure that the bonding contact resistance between the antenna drive (2) and the fuselage structure is not higher than 5 milliohms (Ref. AMM TASK 20-28-00-912-810).

- Apply Varnish Electrical Bonding Structure (Material No. 04TMB2) to the sealant on the head of the screws (5).

NOTE: It is not mandatory to paint the sealant around the base of the antenna and on the head of the screws.

- Remove the blanking caps from the electrical connectors and from the flange of the wave-guide (4).

- Put the new IPC -CSN (34-41-41-01-040) O-ring (3) in the correct position on the flange of the waveguide (4).

- Connect the wave-guide (4) to the antenna drive (2) with the quick disconnect clamp (5).

- Connect the electrical connectors (12).

Subtask 34-41-11-420-051-A B.    Installation of the Antenna

NOTE: Two persons are necessary to do this job.

- Attach the antenna (1) to the ring mount (15) with the screws (17) and the lockwashers (16).

- Torque the screws (17) to between 0.23 and 0.26 m.daN (20.35 and 23.01 lbf.in).

- Connect the flange of the waveguide (14) to the antenna (1).

- Install the screws (18) and the washers (19).

- Make sure that the antenna turns correctly in azimuth (AZ) and in elevation (EL), examine the EL and AZ switches (9).

Subtask 34-41-11-865-052-A

C.   Remove the safety clip(s) and the tag(s) and close this(these) circuit breaker(s):

| PANEL | DESIGNATION | FIN | LOCATION |
| --- | --- | --- | --- |
| 721VU | WXR 1 | 9SQ1 | K03 |
| 722VU | WXR 2 | 9SQ2 | E49 |

Subtask 34-41-11-730-050-A

- Do the operational test of the weather radar antenna scanning

(Ref. AMM TASK 34-41-00-710-802).

5.    Close-up

Subtask 34-41-11-410-050-A

A.   Close Access

(a)  Make sure that the work area is clean and clear of tools and other items.

(b)  Close the access door 811 and the radome 110AL (Ref. AMM TASK 53-15-11-410-801).

(c)  Remove the access platform(s).

(d)   Remove the warning notice(s).

Subtask 34-41-11-862-050-A

B.   De-energize the ground service network

(Ref. AMM TASK 24-42-00-862-801).

ANNEXE 8

TASK 24-41-00-861-801-A

Energize the Aircraft Electrical Circuits from the External Power A

CAUTION:

MAKE SURE THAT THE CONNECTOR OF THE GROUND POWER UNIT IS CORRECTLY CONNECTED TO THE EXTERNAL POWER RECEPTACLE BEFORE YOU START THE GROUND POWER UNIT.

THERE IS A RISK OF ARCING IF THE CONNECTOR IS NOT CORRECTLY CONNECTED.

- Energize the Aircraft Electrical Circuits from the External Power

(1) On the ELEC control panel 235VU, make sure that:

·   The GEN 1, GEN 2, GEN 3, GEN 4, APU GEN, BUS TIE, AC ESS FEED and COMMERCIAL pushbutton switches are pushed in,

·   All the other pushbutton switches (PBSW) are released.

(2) On the VENTILATION section of the panel 212VU, make sure that:

·   The EXTRACT and CAB FANS pushbutton switches are pushed in.

(3) Obey the instructions given with the ground power unit and start the ground power unit.

(4) If the protection circuits operate correctly:

·   On the EXT PWR control panel 925VU, the EXT PWR A/NOT IN USE and EXT PWR A/AVAIL indicator lights come on

·   On the ELEC control panel 235VU, the AVAIL legend of the EXT A pushbutton switch comes on.

(5) On the ELEC control panel 235VU, push the EXT A pushbutton switch:

·   The AVAIL legend goes off,

·   The ON legend comes on.

(6) On the ELEC control panel 235VU, these legends come on:

·   The FAULT legend of the GEN 1, GEN 2, GEN 3 and GEN 4 pushbutton switches,

·   The OFF legend of the BAT 1, BAT 2 and APU BAT pushbutton switches,

·   The OFF legend of the GALLEY pushbutton switch.

(7) On the EXT PWR control panel 925VU:

·   The EXT PWR A/NOT IN USE indicator light goes off.

(8)  On the VENTILATION section of the panel 212VU:

·   Make sure that the FAULT legend of the EXTRACT pushbutton switch is off (this shows that the electronics rack ventilation operates correctly).

(9)  Do the EIS start procedure (EWD DU, SD DU only) (Ref. AMM TASK 31-60-00-860-801).

(10) On the ECAM control panel, push the EL/AC pushbutton switch and make sure that, on the SD:

·   EXT A supplies busbars AC2-3 an AC2-4.

(11) On the ELEC control panel 235VU, push the BAT 1, BAT 2 and APU BAT pushbutton switches:

· The OFF legends go off.

- Controls and Indications on Panels 235VU et 925VU

As soon as the ground power unit A (B) is connected and selected on, the GAPCU analyzes the voltage delivered to the external power receptacle.

If the analyzed parameters (voltage, frequency, phase) are correct, the GAPCU controls:

· The illumination of the green AVAIL legend of the ELEC/EXT A (B) pushbutton switches 10XG (17XG) located on the overhead ELEC panel (235VU)

· The illumination of the amber EXT PWR A (B) AVAIL indicator lights 8XG (32XG) and the white EXT PWR A (B) NOT IN USE indicator lights 7XG (31XG) located in the external power receptacle housing.

______________________

ANNEXE 9

**ON A/C ALL

GÉNÉRATION ÉLECTRIQUE

ALIMENTATION DE L’E/R RADAR MÉTÉO WRX1

3XN1

AC BUS 1

1XP2

ANNEXE 10

TASK 34-41-00-710-802-A

Operational Test of the Weather-Radar Antenna Scanning

WARNING: OBEY THESE SAFETY PRECAUTIONS WHEN THE WEATHER RADAR IS IN OPERATION.

MAKE SURE THAT:

- NO PERSON IS IN THE AREA MADE BY AN ARC OF 135 DEG. ON EACH SIDE OF THE AIRCRAFT CENTERLINE.

- THERE IS A MINIMUM SAFETY DISTANCE OF 60 METERS (200FT) BETWEEN THE ANTENNA

AND ALL REFUELING OPERATIONS.

- THERE IS A MINIMUM SAFETY DISTANCE OF 5 METERS (16.4 FT) BETWEEN THE AIRCRAFT

AND ANY OBSTACLE IN THE AREA MADE BY AN ARC OF 90° ON EACH SIDE OF THE AIRCRAFT CENTERLINE.

THE AIRCRAFT MUST NOT BE IN A HANGAR OR CLOSED AREA.

1.    Reason for the Job

Self-explanatory

2     Job Set-up Information

- Work Zones and Access Panels

| ZONE/ACCESS | ZONE DESCRIPTION |
| --- | --- |
| 110 | RADOME |
| 210 | CKPT FWD PRESSURE BHD TO CKPT PARTITION |
| 811 | FORWARD AVIONICS COMPARTMENT DOORS |
| 110AL, 811 |  |

B.   Referenced Information

| REFERENCE | DESIGNATION |
| --- | --- |
| 24-41-00-861-801-A | Energize the Aircraft Electrical Circuits from the External Power A |
| 24-41-00-861-801-A-01 | Energize the Aircraft Electrical Circuits from the APU (APU Started with APU BAT) |
| 24-41-00-861-801-A-03 | Energize the Aircraft Electrical Circuits from the External Power A and B |
| 24-41-00-861-801-A-04 | Energize the Aircraft Electrical Circuits from the APU (APU started with External power A) |
| 24-41-00-861-801-A-05 | Energize the Aircraft Electrical Circuits from the External Power B |
| 24-41-00-862-801-A | De-energize the Aircraft Electrical Circuits from the External Power A |
| 24-41-00-862-801-A-01 | De-energize the Aircraft Electrical Circuits from the APU |
| 24-41-00-862-801-A-03 | De-energize the Aircraft Electrical Circuits from the External Power A and B |
| 24-41-00-862-801-A-04 | De-energize the Aircraft Electrical Circuits from the External Power B |
| 45-10-00-860-804-A | Procedure to get access to the SYSTEM REPORT/TEST NAV:RADIO NAV page |

3.    Job Set-up

Subtask 34-41-00-860-054-D

A.   Aircraft Maintenance Configuration

- Energize the aircraft electrical circuits

(Ref. AMM TASK 24-41-00-861-801).

- On the center pedestal, on the weather radar control unit, set:

·   The SYS1/OFF/SYS2 switch to OFF

·   The GAIN potentiometer to CAL

·   The TILT selector switch to 0

·   The mode selector switch to WX

·   The MULTISCAN MAN/AUTO switch to MAN

·   The GCS OFF/AUTO switch to OFF.

B.   Remove the safety clip(s) and the tag(s) and close this(these) circuit breaker(s):

| PANEL | DESIGNATION | FIN | LOCATION |
| --- | --- | --- | --- |
| 721VU | WXR 1 | 9SQ1 | K03 |
| 722VU | WXR 2 | 9SQ2 | E49 |

4.    Procedure

Subtask 34-41-00-710-050-C

A.   Operational Test of the Weather-Radar Antenna Scanning

NOTE: This test is for the weather radar 1.

| ACTION | RESULT |
| --- | --- |
| 1.On the center pedestal, on the weather-radar control unit: Set the SYS1/OFF/SYS2 switch to SYS1. |  |
| 2.On the Multipurpose Control and Display Unit (MCDU): · Push the line key adjacent to the WXR 1 Indication. | · the WXR 1 (WXR 2) page comes into view. |
| 3.Push the line key adjacent to the TEST indication | · the TEST IN PROGRESS indication comes into view · during the test sequence, make sure that the antenna: . turns in elevation (+15°, -15°) . turns in azimuth from the right side to the left side, then becomes stable at 0° , vertical to the aircraft centerline. · at the end of the test, the TEST OK indication comes into view. NOTE: If the test is performed with WXR 2, you must energize the WXR 2 when indication IF TEST PATTERN REQUIRED-SWITCH TO SYS- TEM 2 comes into view on the MCDU. |
| 4.Push the line key adjacent to the RETURN indication. | · the antenna scanning stops. |

WIRING DIAGRAM DE L’INSTALLATION RADAR MÉTÉOROLOGIQUE

ANNEXE 11

SYNOPTIQUE DE L’INSTALLATION RADAR MÉTÉOROLOGIQUE

ANNEXE 12

**ON A/C ALL

ANNEXE 13 (1/2)

** ON A/C FSN ALL Technical Data related to a Wire Wire Identification Wire Number : 3441-0047 Color : B Type : TT Gauge : 24 Wire Part Number : EN2714-011B002F Length : 274 cm Route : 1M Diagram Ref. : Ref. AWM 34-41-02 Feed Thru : 1VP / Bundle : 1055VB Bundle Diagram Ref. : Ref. AWM 91-01-55 From Termination A FIN : 7SQ Connector : 7SQA Contact Number : <Q Contact Part Number : EN3155-019F2018 To Termination B FIN : 515VC Connector : 515VCA Contact Number : 18 Contact Part Number : EN3155-015F2018

** ON A/C FSN ALL Technical Data related to a Wire Wire Identification Wire Number : 3441-0047 Color : R Type : TT Gauge : 24 Wire Part Number : EN2714-011B002F Length : 274 cm Route : 1M Diagram Ref. : Ref. AWM 34-41-02 Feed Thru : 1VP / Bundle : 1055VB Bundle Diagram Ref. : Ref. AWM 91-01-55 From Termination A FIN : 7SQ Connector : 7SQA Contact Number : <P Contact Part Number : EN3155-019F2018 To Termination B FIN : 515VC Connector : 515VCA Contact Number : 17 Contact Part Number : EN3155-015F2018

AWM

NAVIGATION WEATHER RADAR SYS 1 -ANTENNA

ANNEXE 13 (2/2)

|  | OUTIL 1 |
| --- | --- |
|  | OUTIL 2 |
|  | OUTIL 3 |

ANNEXE 14

1.  Power Supply

Energization of each system is through 115VAC 400 Hz buses:

·   1XP via the busbar 401 XP-A for transceiver 1

·   2XP via the busbar 204 XP-A for transceiver 2.

Energization of the Weather Radar Control Unit and the WXR antenna drive is through the selected transceiver. Consumption of each selected transceiver is 150W nominal.

The system is supplied through this (these) circuit breaker (s):

-------------------------------------------------------------------------------------------------------------------

PANEL 	DESIGNATION		FIN 			LOCATION

-------------------------------------------------------------------------------------------------------------------

721VU 		WXR 1 			9SQ1 			K03

722VU 		WXR 2 			9SQ2 			E49

2.  Interface

The weather radar data are transmitted via ARINC 429 and 453 buses. The table below contains all the output parameters in the digital form.

-------------------------------------------------------------------------------

|     PARAMETER LIST                  PARAMETER CHARACTERISTICS (NUMERIC)     |

-------------------------------------------------------------------------------

|EQ.SYS.LAB.SDI|PARAMETER | WORD RANGE |UNIT|SIG |BITS|XMSN|CODE|ALPHA|SOURCE |

|              |DEFINITION| OPER RANGE |    |BIT |    |INTV|    |CODE |ORIGIN |

|              |(*=REMARK)| RESOLUTION |    |    |    |    |    |     |BUS No.|

|              |(X=NOTE)  |  ACCURACY  |    |    |    |    |    |     |ATA REF|

|              |          |            |    |    |    |    |    |     |CONV   |

-------------------------------------------------------------------------------

!       270    ! CONTROL  !    16      ! deg!    ! 4  !100 ! BNR!     !       !

!              ! WORD 1   !    0-15    !    !    !    !    !    !     !       !

!              !          !    1       !    !    !    !    !    !     !       !

!              !          !    0,5     !    !    !    !    !    !     !       !

!--------------!----------!------------!----!----!----!----!----!-----!-------!

!       271    ! CONTROL  !            !    !    ! 32 !100 ! DIS!     !       !

!              ! WORD 2   !            !    !    !    !    !    !     !       !

!--------------!----------!------------!----!----!----!----!----!-----!-------!

!       055    ! WXR DATA !            !    !    !1600!7.82! DIS!     !       !

!--------------!----------!------------!----!----!----!----!----!-----!-------!

- Control Words (Label 270 and 271)

The control words are transmitted on the control buses (ARINC 429) which connect the weather radar control unit to the transceivers (one for each transceiver).

The control word 1 (label 270) provides the following data:

·   TILT angle

·   The SSM specifies the TILT angle sign

Selected Mode:

| Bit 25 | Bit 26 | Mode |
| --- | --- | --- |
| 0 | 0 | WX |
| 0 | 1 | WX + T |
| 1 | 0 | Mode Turb |
| 1 | 1 | MAP |

Digital Interface Special for WXR/PWS

Sign / Status Matrix (SSM) encoded BNR:

| Bit 31 | Bit 30 | Signification |
| --- | --- | --- |
| 0 | 0 | Plus, North, East, Right, To, Above |
| 0 | 1 | No computed data |
| 1 | 0 | Functionnal test |
| 1 | 1 | Minus, South, West, Left, From, Below |

The control word 2 (label 271) provides the following data:

·   Selected mode			bit 11-12

·   Stabilization (on/off)	bit 13 / Discrete (On: 1 / off: 0)

·   scan (normal/reduced)	bit 14 / Discrete (Norm.:1 / Red.:0)

·   Anti-clutter (on/off).	bit 15 / Discrete (On: 1 / off: 0)

·   Range 									bit 16-27

·   Anti-clutter (on/off) 						bit 28

·   WXR selected on the EFIS control panel (master/slave) 	bit 29

3. Data Word (Label 055)

The signal transmitted on the data bus line, which connects the WXR/PWS to the Display Management Computers (DMC), complies with data word format (label 055) as defined by the ARINC Specification 708A (Appendix 15.2).

The following table defines the label 055:

| BITS | FUNCTION |
| --- | --- |
| 1-8 | Label 00101101: 055 |
| 9-10 | Control Accept |
| 11 | Spare |
| 12 | Windshear Caution |
| 13 | Windshear Warning |
| 14 | Turbulence Alert (not used) |
| 15 | Tilt Selection |
| 16 | Anti-clutter (not used) |
| 17 | Sector Scan |
| 18 | Stabilization Limits |
| 19 | Cooling Transceiver |
| 20 | Display Fault (not used) |
| 21 | Calibration or Air Data Input |
| 22 | Attitude Input |
| 23 | Control or Heading Input |
| 24 | Antenna |
| 25 | R/T or Radio Altimeter |
| 26 | Stabilization |
| 27-29 | Mode |
| 30-36 | Tilt Angle |
| 37-42 | Gain |
| 43-48 | Range |
| 49 | Windshear External Input Fault |
| 50-51 | Data Accept |
| 52-63 | Scan Angle |
| 64 | Windshear Failure |
| 65-1600 | Range Bin Data |

The data bus line transmits 190 data words per second; each word is made up of 1600 bits sent on 1 Megabit/s frequency (ARINC Specification 453) as per MANCHESTER II BI PHASE 2 format.

ANNEXE 15

Oscillogramme 2

Oscillogramme 1

Exemple de lecture d’une trame ARINC 429 relevée à l’oscilloscope :

Mot binaire : 10(2)

Trame correspondante :

Oscillogramme 3

Oscillogramme 4

ANNEXE 16

TASK 34-41-00-810-803

Failure of the Weather Radar Antenna (Circuit 1)

1.  Possible Causes

- DRIVE-WXR   ANTENNA (7SQ)

- wiring from the weather radar antenna drive to the WXR transceiver 1

2.  Job Set-up Information

- Referenced Information

| REFERENCE |  | DESIGNATION |
| --- | --- | --- |
| AMM | 34-41-00-740-801 | BITE test of the Weather Radar |
| AMM | 34-41-11-000-801 | Removal of the Weather Radar Antenna Assembly (7SQ, 11SQ) |
| AMM | 34-41-11-400-801 | Installation of the Weather Radar Antenna Assembly (7SQ, 11SQ) |
| AMM | 34-41-33-000-801 | Removal of the Weather Radar Transceiver (1SQ1, 1SQ2) |
| AMM | 34-41-33-400-801 | Installation of the Weather Radar Transceiver (1SQ1, 1SQ2) |
| ASM | 34-41/01 |  |

3.  Fault Confirmation

A.  Test

Do the BITE test of the weather radar 1 (Ref.  AMM TASK   34-41-00-740-801).

4.  Fault Isolation

R     **ON A/C 001-099, 101-155, 201-299, 501-549, 551-599,

A.  If the test gives the maintenance message   RADAR 1 ANTENNA:

- set the system 2 on the WXR control unit.

(1)  If the fault continues:

- Replace the DRIVE-WXR   ANTENNA (7SQ) (Ref.  AMM TASK   34-41-11-000-801) and (Ref.  AMM TASK   34-41-11-400-801)

- set the system 1 on the WXR radar control unit.

(a)  If the fault continues:

- remove the WXR transceiver 1 (Ref.  AMM TASK   34-41-33-000-801)

- remove the WXR Antenna drive

- do a check and repair the wiring from the weather radar antenna drive to the WXR transceiver 1:  from connector A to connectors AB and AC (Ref.  ASM 34-41/01)

R     **ON A/C 156-199, 301-349,

A.  If the test gives   the maintenance message WXR ANTENNA   (11SQ):

- set the system 2 on the weather radar control unit.

(1)  If the fault continues:

- Replace the DRIVE-WXR   ANTENNA (7SQ) (Ref.  AMM TASK   34-41-11-000-801) and (Ref.  AMM TASK   34-41-11-400-801)

- set the system 1 on the weather radar control unit.

(a)  If the fault continues:

- remove the weather radar transceiver 1 (Ref.  AMM TASK   34-41-33-000-801)

- remove the WXR antenna drive

- do a check and repair the wiring from the WXR antenna drive to the

WRX transceiver 1:  from connector A to connectors AB and AC (Ref.  ASM 34-41/01)

- install the WXR transceiver 1 (Ref.  AMM TASK   34-41-33-400-801)

- install the WXR antenna drive.

**ON A/C ALL

B.  Do the test given in Para.  3.