Le dossier technique se compose de 17 pages, numérotées de 1/17 à 17/17.

Dès que le dossier technique vous est remis, assurez-vous qu’il est complet.

DOSSIER TECHNIQUE

BaccalaurÉat Professionnel

AÉRONAUTIQUE

OPTION : AVIONIQUE

ÉPREUVE E2 – EXPLOITATION DE LA DOCUMENTATION TECHNIQUE

CODE :1806-AER A U2

Table des matières

1-	INTRODUCTION:	2

1.	Définition et surfaces de contrôle	2

2.	Commandes de vol primaires	2

3.	Commandes de vol secondaires	2

2-	PRÉSENTATION – SPOILER	3

2.	Principe/conception :	4

3.	Actionneurs :	4

4.	Ground Spoiler Actuation	5

3-	TROUBLESHOOTING PRINCIPLE (TSM).	6

TASK HOA2-A-27-64-XX-5R001-421A-A	6

TASK HOA2-A-27-64-XX-1U002-429A-A	7

4-	REMOVE AND INSTALL	8

TASK HOA2-A-27-93-34-00001-520A-A	8

5-	CARACTERISTIQUES EBHA:	14

6-	CARACTERISTIQUES SERVO-CONVENTIONELLE:	14

7-	ANNEXE 1	15

8-	ANNEXE 2	16

INTRODUCTION:

En sortie de chaîne de production, l’aéronef de type HOA2 présente un message d’erreur de type « EBHA-R SPLR 5, Y (5CL2) lors du test FCGS. L’étude portera sur le spoiler 5 EBHA.

Définition et surfaces de contrôle

Les commandes de vol sont l’ensemble des éléments de pilotage destinés à maitriser(ou commander) le vol de l’avion.

Ces éléments sont :

-Les organes de pilotage,

-La chaine de commande ou interfaces

-Les surfaces mobiles

Commandes de vol primaires

Permettre de contrôler l'attitude, donc la trajectoire et la vitesse en vol.

Les gouvernes concernées sont :

-1:Aileron externe ou "basse vitesse"

2

2

1

1

3

4

5

6

6

-2:Aileron interne ou "toute vitesse"

-3:Gouverne de profondeur

-4:Gouverne de direction

-5:PHR

-6:Spoiler

Commandes de vol secondaires

Le système de commandes de vol secondaires est l'ensemble des éléments destinés à modifier les caractéristiques aérodynamiques de l'avion.

9

8

7

Il s'agit de la commande des hyposustentateurs et hypersustentateurs (becs, volets et spoilers) ainsi que des aérofreins, que l’on peut retrouver aussi sur le fuselage.

-7:Becs mobiles

-8:Volets

-9:Spoilers

-10:Aérofreins

9

PRÉSENTATION – SPOILER

- Description

Position des spoilers 1 (1CL1/2), 2 (2CL1/2) et 7 (7CL1/2) sont commandés par le système hydraulique  jaune, les spoilers 3 (3CL1/2), 4 (4CL1/2) et 6 (6CL1/2) sont commandées par le système hydraulique vert. Le spoiler 5 (5CL1/2) est commandé électriquement et actionnée par l’hydraulique jaune.

Principe/conception :

Sept spoilers sur chaque aile actionnées par des actionneurs. Chaque actionneur de spoiler est électriquement commandé/contrôle par des calculateurs (PRIM et SEC) de levier « SPEED BRACK », et du mini-manche (SSUs CAPT&F/O) ou du pilote automatique. Tous les spoilers sont à commande hydraulique par des actionneurs conventionnels de servocommande, excepté la position 5, celle-ci est déclenchée par un vérin hydraulique de secours électrique (EBHA). Chaque actuator/EBHA est commandé et surveillé par les calculateurs PRIM et SEC. La surface et le déclencheur placent la rétroaction envoyée aux PRIM et SEC pour le calcul de position de la servocommande.

Actionneurs :

L’aéronef emploie différents types d’actionneurs :

Ground Spoiler Actuation

TROUBLESHOOTING PRINCIPLE (TSM).

### TASK HOA2-A-27-64-XX-5R001-421A-A

Fault of the Right Spoiler 5 EBHA: code panne “ EBHA-R SPLR 5, Y (5CL2)”

- Fault Confirmation

- Go to the Fault Isolation.

- Check the value of the data of label 167 of ARINC data bus 429 between the PRIM and SEC 
Note: Value of the data must lie between 80 and 100° deg.

- Check the discrete state of SPOILER 5CL2 to 28V => “Ground Actuation Spoiler”

Go to the Fault Confirmation.

Does the fault continue? YES (Go to Step C.) / NO (Go to Step F.)

- Remove PRIM1.( Ref:annexe 1)

Disconnect electrical connector E41 from EPDC1 (1XZ).

Do a check and repair the wiring of the FCGS_DIS_R_SPLR5_EBHA_CMD_M signal

Value of continuity: between 15Ω and 25Ω.

Do a check and repair the wiring of the FCGS_DIS_R_SPLR5_EBHA_CMD_C signal ASD HOA2-A-27-64-XX-03ZZZ-054Z-A

Value of continuity: between 30Ω and 40Ω.

Connect electrical connector E41 to EPDC1 (1XZ).

Install PRIM1.

Go to the Fault Confirmation .

Does the fault continue? YES (Go to Step D.) / NO (Go to Step F.)

- Remove SEC1.

Disconnect electrical connector E41 from EPDC1 (1XZ).

Do a check and repair the wiring of the FCGS_DIS_R_SPLR5_EBHA_CMD_M signal

Value of continuity: between 15Ω and 25Ω.

Do a check and repair the wiring of the FCGS_DIS_R_SPLR5_EBHA_CMD_C signal Ref. ASD HOA2- A-27-64-XX-03ZZZ-054Z-A:

Value of continuity: between 20Ω and 30Ω.

Connect electrical connector E41 to EPDC1 (1XZ).

Install SEC1.

Go to the Fault Confirmation .

Does the fault continue? YES (Go to Step E.) / NO (Go to Step F.)

- Disconnect electrical connector X65 from EPDC1 (1XZ).

Disconnect electrical connector P from the right spoiler 5 EBHA (5CL2).

Do a check and repair the wiring of the EPDC_1_PSY_R_SPLR5_EBHA signal.

Value of continuity: between 25Ω and 35Ω.

Connect electrical connector P to the right spoiler 5 EBHA (5CL2).

Connect electrical connector X65 to EPDC1 (1XZ).

Go to Step F.

- Go to the Close-up.

Go to Step G.

- Replace the RCCB-EBHA SPLR5 R (2043CL)

Go to the Fault Confirmation

Does the fault continue? YES (Go to Step H.) / NO (Go to Step 3.)

- Replace the ELEK MODULE-EBHA ,R SPLR 5 Y (405CL2) Ref.

- Go to the Fault Confirmation .

Does the fault continue? YES (Go to Step J.) / NO (Go to Step 3.)

- Replace the EBHA-R SPLR 5,Y (5CL2)

- Go to the Fault Confirmation

. Does the fault continue? YES (Go to Step A.) / NO (Go to Step 3.)

- Close-Up

Put the aircraft back to its initial configuration.

Make sure that the work area is clean and clear of tools and other items.

_______________________________

### TASK HOA2-A-27-64-XX-1U002-429A-A

Fault of the Left Spoiler 1 FCRM Detected by the SEC 1 Monitoring

- Fault Confirmation

Go to the Fault Isolation

  - Replace the SERVO MODULE-SERVO CTL,L SPLR1 Y (201CL1)

Go to the Fault Confirmation

Does the fault continue? YES (Go to Step B.) / NO (Go to Step L.)

  - Replace the SERVO CTL-L SPLR 1,

Go to the Fault Confirmation

Does the fault continue? YES (Go to Step C.) / NO (Go to Step L.)

  - Replace SEC-1 (2CE1)

Go to the Fault Confirmation .

Does the fault continue? YES (Go to Step D.) / NO (Go to Step L.)

  - Remove PRIM1.

Do a check and repair the wiring of the FCGS_ANA_SRVCTL_L_SPLR1_Y signal

Install PRIM1.

Go to the Fault Confirmation

Does the fault continue? YES (Go to Step E.) / NO (Go to Step L.)

- Replace the COUPLER-LW COM 1 (105CE1)

Go to the Fault Confirmation

Does the fault continue? YES (Go to Step F.) / NO (Go to Step L.)

- Replace the COUPLER-LW MON 1 (105CE2).

Go to the Fault Confirmation

Does the fault continue? YES (Go to Step G.) / NO (Go to Step L.)

- Disconnect electrical connector B from the coupler of the 1553 bus (105CE1)

Disconnect electrical connector A from the Yellow left spoiler-servocontrol (1CL1).

Do a check and repair the wiring of the FCGS_1553_1_L_WG_SVCTRL_P1S1C signal

Connect electrical connector A to the Yellow left spoiler-servocontrol (1CL1). Connect electrical connector B to the coupler of the 1553 bus (105CE1). Go to the Fault Confirmation

Does the fault continue? YES (Go to Step H.) / NO

(Go to Step L.)

- Disconnect electrical connector B from the coupler of the 1553 bus (105CE2). Disconnect electrical connector B from the Yellow left spoiler-servocontrol (1CL1).

Do a check and repair the wiring of the FCGS_1553_2_L_WG_SVCTRL_P1S1M signal

Connect electrical connector B to the Yellow left spoiler-servocontrol (1CL1).

Connect electrical connector B to the coupler of the 1553 bus (105CE2).

Go to the Fault Confirmation

Go to Step L.

- Go to the Close- up.

Go to Step 2.

- Close-Up

Put the aircraft back to its initial configuration.

Make sure that the work area is clean and clear of tools and other items.

REMOVE AND INSTALL

- TASK HOA2-A-27-93-34-00001-520A-A

Removal of the PRIMary Computer (PRIM)

FIN: 1CE1 1CE2 1CE3

- General

  - Reason for the Job Self Explanatory

  - Component Weight

The weight of the component is approximately 12 kg (26.5 lb).

- Job Set-Up Information

A.	Work Zones.

- Job Set-Up

SUBTASK 279334-10100010001

- Aircraft Maintenance Configuration

  - Energize the aircraft electrical circuits

Ref. MP HOA2-A-24-41-XX-00ZZZ-761Z-A .

  - On a maintenance terminal, get access to the “MAINTENANCE HOME PAGE” Ref. MP HOA2-A-45-XX-XX-00ZZZ-132Z-A .

  - On the “MAINTENANCE HOME PAGE”, select “C/B Management” Ref. MP HOA2-A-24-71-XX-00ZZZ-170Z-A

SUBTASK 279334-86500010001

- Open, safety/lock and tag this(these) circuit breaker(s):

SUBTASK 279334-10100020001

- Get access to the avionics compartment

Ref. MP HOA2-A-12-XX-XX-00ZZZ-170Z-A

- Procedure

SUBTASK 279334-40100010001

- Removal of the PRIM

  - Loosen and lower the nuts (4).

WARNING:	BE CAREFUL WHEN YOU REMOVE OR INSTALL THIS EQUIPMENT. THIS EQUIP-MENT IS HEAVY (MORE THAN 12 KG (26.5 LB)) AND CAN CAUSE INJURY AND/

OR DAMAGE.

- Carefully pull the PRIM (1) on its rack (3) to disconnect the electrical connectors (2) and remove it.

- Put a cap-blanking on all the disconnected electrical connector(s) and receptacle(s). CAP -

BLANKING.

#### TASK HOA2-A-27-94-34-00001-520A-A

Removal of the SECondary Computer (SEC)

FIN: 2CE1 2CE2 2CE3

- General

  - Reason for the Job Self Explanatory

  - Component Weight

The weight of the component is approximately 12 kg (26.5 lb).

- Job Set-Up Information

HOA2-A-12-XX-XX-00ZZZ-170Z-A: MP - Get Access / Close Access to the Avionics Compartment

MP - Energize the Aircraft Electrical Circuits from External Power Re- ceptacle 1

HOA2-A-24-41-XX-00ZZZ-761Z-A

HOA2-A-24-71-XX-00ZZZ-170Z-A: MP - Get Access to "C/B Management"

MP - Get Access to the "MAINTENANCE HOME PAGE" and to the

HOA2-A-45-XX-XX-00ZZZ-132Z-A : Maintenance Applications

- Job Set-Up

SUBTASK 279434-10100010001

- Aircraft Maintenance Configuration

  - Energize the aircraft electrical circuits

Ref. MP HOA2-A-24-41-XX-00ZZZ-761Z-A .

  - On a maintenance terminal, get access to the “MAINTENANCE HOME PAGE” Ref. MP HOA2-A-45-XX-XX-00ZZZ-132Z-A .

  - On the “MAINTENANCE HOME PAGE”, select “C/B Management” Ref. MP HOA2-A-24-71-XX-00ZZZ-170Z-A .

SUBTASK 279434-86500010001

B.	Open, safety/lock and tag this(these) circuit breaker(s):

SUBTASK 279434-10100020001

- Get access to the avionics compartment

Ref. MP HOA2-A-12-XX-XX-00ZZZ-170Z-A .

- Procedure

Ref. Fig. SEC

SUBTASK 279434-40100010001

- Removal of the SEC

  - Loosen and lower the nuts (4).

WARNING:	BE CAREFUL WHEN YOU REMOVE OR INSTALL THIS EQUIPMENT. THIS EQUIP-MENT IS HEAVY (MORE THAN 12 KG (26.5 LB)) AND CAN CAUSE INJURY AND/

OR DAMAGE.

- Pull the SEC (1) on its rack (3) to disconnect the electrical connectors (2) and remove it.

- Put a cap-blanking on all the disconnected electrical connector(s) and receptacle(s). CAP -

BLANKING.

#### TASK HOA2-A-27-64-55-A2001-520A-A

Removal of the Electronic Module of the Spoiler Electrical Backup Hydraulic Actuator (EBHA)

FIN: 405CL1 405CL2

- General

  - Reason for the Job Self Explanatory

  - Additional information

Two persons are necessary to do this procedure (one person to hold the electronic module and one per-son to loose the captive screws).

  - Component Weight

The weight of the electronic module is approximately 12 kg (26 lb).

- Job Set-Up Information

Access/Panel.

ON A/C 0088

FOR FIN 405CL1 (ELEK MODULE-EBHA,L SPLR 5 Y)

572DB (Panel)

ON A/C 0088

FOR FIN 405CL2 (ELEK MODULE-EBHA ,R SPLR 5 Y)

672DB (Panel)

ON A/C 0088

Fixtures, Tools, Test and Support Equipment.

Referenced Information.

- Job Set-Up

SUBTASK 276455-10100280001

- Aircraft Maintenance Configuration

  - Energize the aircraft electrical circuits

Ref. MP HOA2-A-24-41-XX-00ZZZ-761Z-A .

  - Extend the flaps Ref. MP HOA2-A-27-5X-XX-00ZZZ-170Z-A or Ref. MP HOA2-A-27-5X-XX-04ZZZ-170Z-A or Ref. MP HOA2-A-27-5X-XX-06ZZZ-170Z-A .

  - Put the hydraulic systems in the depressurized-for-maintenance configuration Ref. MP HOA2-A-29-XX-XX-05ZZZ-562Z-A .

  - On a maintenance terminal, get access to the “MAINTENANCE HOME PAGE” Ref. MP HOA2-A-45-XX-XX-00ZZZ-132Z-A .

  - On the “MAINTENANCE HOME PAGE”, select “C/B Management” Ref. MP HOA2A-24-71-XX-00ZZZ-170Z-A .

SUBTASK 276455-86500100001

- Open, safety/lock and tag this(these) circuit breaker(s):

SUBTASK 276455-10100460001

- Aircraft Maintenance Configuration

  - On the EPDC1 inner face:

    - On location 1CF04, open reset switch 1663XZ.

      - On the “C/B Management” page, make sure that RCCB 2041CL is opened.

    - On location 1CG03, open reset switch 1657XZ.

      - On the “C/B Management” page, make sure that RCCB 2043CL is opened.

SUBTASK 276455-10100290001

- Safety Precautions

  - On the CAPT and F/O side-stick consoles, on each SSU:

    - Install the PIN,LOCKING-SIDE STICK (98L27901001000).

    - Put a WARNING NOTICE(S) in position to tell persons not to operate the CAPT and F/O SSUs.

  - On the center pedestal, on the FLAPS control panel:

    - Install the CONTROL LEVER FLAP SLAT (98V27801000000) on the SLATS/FLAPS control lever.

    - Put a WARNING NOTICE(S) in position to tell persons not to operate the SLATS/FLAPS control lever.

  - On the overhead panel, on the F/CTL sections of panels 211VM and 212VM, put the WARNING NOTICE(S) in position to tell persons not to operate the flight controls.

  - On the overhead panel, make sure that the WARNING NOTICE(S)WARNING NOTICE(S) is(are) in position to tell persons not to pressurize the Green and Yellow hydraulic systems, on:

    - The GND HYD section of panel 222VM

    - The HYD section of panel 235VM.

  - Make sure that the WARNING NOTICE(S) is(are) in position on the Green and Yellow ground service-panels (access doors 194KB and 197LB) to tell persons not to operate the hydraulic systems WARNING NOTICE(S).

  - As necessary, use the applicable SAFETY BARRIER(S)specified by the operator instructions and your local regulations.

SUBTASK 276455-10100300001

- Get Access

  - Put the ACCESS PLATFORM 6M (20 FT) - ADJUSTABLE in position:

FOR FIN 405CL1 (ELEK MODULE-EBHA,L SPLR 5 Y)

| (a) Below access panel 572DB. |  |
| --- | --- |

FOR FIN 405CL2 (ELEK MODULE-EBHA ,R SPLR 5 Y)

- Below access panel 672DB.

ON A/C 0088

WARNING:	BEFORE YOU START WORK, YOU MUST PUT ON A SAFETY HARNESS AND AT-TACH IT TO THE ACCESS PLATFORM. WITHOUT A SAFETY HARNESS, YOU CAN FALL. THIS CAN KILL YOU OR CAUSE YOU INJURY.

- Put on and attach a SAFETY HARNESS before you start work in the work area.

- Removal for access:

FOR FIN 405CL1 (ELEK MODULE-EBHA,L SPLR 5 Y)

  - Remove access panel 572DB Ref. MP HOA2-A-57-52-71-01ZZZ-520Z-A .

FOR FIN 405CL2 (ELEK MODULE-EBHA ,R SPLR 5 Y)

  - Remove access panel 672DB Ref. MP HOA2-A-57-52-71-01ZZZ-520Z-A .

ON A/C 0088

- Procedure

Ref. Fig. Electronic Module of the Spoiler

SUBTASK 276455-40100070001

- Removal of the Electronic Module of the Spoiler EBHA

  - Put on the ELECTRICALLY GROUNDED WRIST STRAP and connect its cable to a bonding point.

  - Install the FLAP DRIVE SHAFT PROTECTOR (98V27004004000) around the flap drive shaft.

WARNING:	BEFORE YOU INSTALL/REMOVE THE GSE TOOL, READ THE MANUFACTURER'S INSTRUCTIONS, AND, WHILE YOU OPERATE THIS TOOL, CAREFULLY OBEY THEM. IF YOU DO NOT DO THIS, INJURY TO PERSONS AND/ OR DAMAGE TO EQUIPMENT CAN OCCUR.

- Install the SPOILER MAINTENANCE KEY (98V27103004000) (3) on the maintenance device

    - of the spoiler EBHA (1).

- Set the maintenance device (2) as follows:

  - With the spoiler maintenance key (3) , push and turn the maintenance device (2) of the spoiler EBHA (1) to the “M” (maintenance) position.

  - Make sure that the maintenance device (2) is against the “M” stop.

NOTE:	The spoiler EBHA is in maintenance mode.

- Extend the spoiler for maintenance Ref. MP HOA2-A-27-64-XX-00ZZZ-170Z-A .

- Removal of the catcher assembly (16) :

  - Remove the sealant with the SCRAPER - NON METALLIC from the catcher assembly (16).

  - Loosen the nuts (17) from the catcher assembly (16).

  - Remove the nuts (17) , the bolts (13) and the washers (14) from the catcher assembly (16).

  - Remove the catcher assembly (16) from the structure (15).

CAUTION:	MAKE SURE THAT ELECTRICAL CONNECTORS ARE CLEAN WHEN YOU DIS-CONNECT THEM. THE CONTAMINATION OF ELECTRICAL CONNECTORS CAN CAUSE DAMAGE TO THE EQUIPMENT.

- Disconnection of the electrical connectors:

  - Put a mark on the electrical connectors (5) , (6) , (7) , (8) , (9) , (10) and (12) and dis-connect them.

- Put a cap-blanking on all the disconnected electrical connector(s) and receptacle(s). CAP -

BLANKING.

- Put the PLUG - BLANKING on each electrical port of the electronic module (4).

- Removal of the electronic module (4) :

    - Hold the electronic module (4) and loosen the captive bolts (11)

WARNING:	BE CAREFUL WHEN YOU REMOVE OR INSTALL THIS EQUIPMENT.

THIS EQUIPMENT IS HEAVY (MORE THAN 12 KG (26.5 LB)) AND CAN CAUSE INJURY AND/OR DAMAGE.

- Carefully remove the electronic module (4) from the spoiler EBHA (1).

CARACTERISTIQUES EBHA:

|  | EBHA |  |  |
| --- | --- | --- | --- |
|  | CA67002-001 | CA67002-010 | CA67002-017 |
| Masse (kg) | 12 | 12 | 12 |
| Pression (PSI) | 5000 | 5000 | 5000 |
| Diamètre piston (mm) | 58 | 62 | 64 |
| Course (mm) | 115 | 118 | 120 |
| Voltage | 230VAC/60VDC | 230VAC/60VDC | 230VAC/60VDC |

CARACTERISTIQUES SERVO-CONVENTIONELLE:

|  | Conventional servocommande |  |  |
| --- | --- | --- | --- |
|  | CA67003-020 | CA67003-020 | CA67003-020 |
| Masse (kg) | 25 | 25 | 25 |
| Pression (PSI) | 5000 | 5000 | 5000 |
| Diamètre piston (mm) | 58 | 62 | 64 |
| Course (mm) | 115 | 118 | 120 |

ANNEXE 1

ANNEXE 2

Contacts câblés : M15570 – 20				Wired contacts : M15570  –  20

référence constructeur    -   taille du contact			    Manufacturer reference    -   Wire