This document describe the “user” functions found in the canapi.lib file

The functions can be classified by type and are discussed below in this manual:

- INIT:

PROTOTYPE		DWORD CAN_Open(VOID);

PURPOSE: 	Open the device file.

The user application must use the CAN_Open function before using any other call.

output parameter:	Return OK or ERROR_CAN_DRIVER (if driver is not loaded).

PROTOTYPE 	DWORD CAN_Close(VOID);

PURPOSE: 		Close the device file and delete FIFO buffer. the user application must close the file driver before leaving.

output parameter:	Return OK or ERROR_CAN_DRIVER .

PROTOTYPE:	DWORD CAN_SetObjectConfig(UNSIGNED CHAR *);

PURPOSE: 		Configure Message Objects direction and configuration .

input parameter :	Pointer to a seven unsigned char table.

output parameter: 	Return OK or CANERROR.

TABLE STRUCTURE: Seven unsigned char must be declared for each channel used, the table must be ended by END_TABLE.

| Byte 1 | Byte 2 | Byte 3 | Byte 4 to 7 |
| --- | --- | --- | --- |
| ChannelNumber | Direction | IdentifierType | ID0,ID1,ID2,ID3 |
| (0- 14) | TRANSMIT RECEIVE NOT_USED | EXTENDED_IDENT STANDARD_IDENT | 4 bytes acceptance Filter OxXX,OxXX,OxXX,OxXX |

The 4 Acceptance Filter Bytes are only relevant for receiving message object.

They must be set according to the Global Mask filter strategy which defines which bit is must-match or is don't care.

Declaration Example :

unsigned char Can_Config[]={

0, TRANSMIT, STANDARD_IDENT, 1,2,3,4,

1, RECEIVE, STANDARD_IDENT, 4, 3,2,1,

END_TABLE};

Use:

CAN_SetObjectConfig(Can_Config) ;

Warning :

As default mode, the driver initialize the 10th first Message Object (0 to 9) as sending channel.

The 15th is used as receiving channel for ALL incoming messages. The message 10 is used as dedicated channel to request Remote Frame.

In this current release, it is only permitted to call this function in order to initialize Message Object 10 to request Remote Frame.

unsigned char Can_Config[]={10, RECEIVE, STANDARD_IDENT,0,0,0,0,END-TABLE};

CAN_SetObjectConfig(Can_Config);

PROTOTYPE 	DWORD CAN_InitBoard(WORD Address, WORD Speed, UNSIGNED CHAR BTR0, UNSIGNED CHAR BTR1 , UNSIGNED CHAR interrupt);

PURPOSE:	Reset the board, perform a light test and set the CAN speed.

input parameters:

First parameter :	Windows NT :Unused (value comes from Registry).

Windows 95 : board address

Speed : 		CAN speed.

BTR0,BTR1 : 	Bit Timing Register values (see 82527 document).

Fifth parameter : 	Windows NT :Unused (value comes from Registry).

Windows 95 : Interrupt resource.

The table indicates the various possible speeds, segments times and Bit Timing Registers values for standard CAN speed.

It is possible to have the same speeds with other values for the BTR0 and BTR1 registers.

| Speed (kb/s) | tSYNCs in s | tTSEG 1 in s | tTSEG2 in s | BRP Hex | BTR0 Hex | BTR1 Hex |
| --- | --- | --- | --- | --- | --- | --- |
| 10 20 50 100 125 250 500 1000 | 5.0 2.5 1.0 0.5 0.5 0.25 0.125 0.125 | 80 40 16 8 6.5 3.25 1.625 0.625 | 1.5 7.5 3 1.5 1 0.5 0.25 0.25 | 27 13 7 3 3 1 0 0 | A7 93 87 83 83 81 80 80 | 2F 2F 2F 2F 1C 1C 1C 14 |

If Speed equal 0: Per default, the Board resets with the SPEED value from the Registry table.

If Speed equal1 : Initialize the speed according to BTR0 and BTR1 parameters.

output parameter: Return OK or CAN_NOBOARD (if board is not recognized) or CANERROR.

- Windows callback

PROTOTYPE 	HANDLE CAN_SetWMes(HWND,UINT MES);

PURPOSE:	Registers the window handle of user's application, initializes a FIFO buffer, registers the Windows message to send to the application in order to indicate that CAN messages are available.

input parameters:

HWND:	Handle of the window to send the 'MES' message.

MES:	Value of the Windows message to send when at least one incoming message becomes available or on error conditions.

output parameter: Handle to FIFO or CANERROR .

CANAPI posts MES message to the HWND window using the PostMessage Windows call.

CANAPI posts MES message when incoming CAN messages are available

the user's application must read ali the available messages to trig the post function again.

CANAPI posts MEs+1 when " CAN warn error" condition occurs on CAN bus

CANAPI posts MEs+2 when " CAN Bus off error" condition occurs on CAN bus

WARNING : Only used for Windows application.

PROTOTYPE 	DWORD CAN_DeleteHandle(HANDLE hw)

PURPOSE:	Delete the FIFO handle of user's application window .

input parameters:

hw: Handle to FIFO butter.

output parameter: Return OK or ERROR_CAN_DRIVER .

- Statistic

PROTOTYPE:	DWORD CAN_GetLostNumber(HANDLE);

PURPOSE: 	This routine returns the number of lost messages as an unsigned DWORD.

input parameter : FIFO handle returned by CAN_SetWMes

output parameter: Count of lost messages since CAN_ClearFIFO was called or CANERROR.

Message lost occurs when a incoming message has been overwritten by a new one or when the fifo gets full.

PROTOTYPE: 	DWORD CAN_GetRecNumber(HANDLE);

PURPOSE: 	This function returns the number of received message.

input parameter : FIFO handle returned by CAN_SetWMes

output parameter: Return the total count of received message since CAN_ClearFIFO() , or CANERROR.

- FIFO Control

PROTOTYPE:	DWORD CAN_GetPendingCount(HANDLE);

PURPOSE: 	This function returns the number of message waiting in the fifo .

input parameter : FIFO handle returned by CAN_SetWMes

output parameter: Number of the remaining messages to read or CANERROR.

PROTOTYPE:	VOID CAN_ClearFIFO(HANDLE);

PURPOSE: 	This routine clears the FIFO buffer, the message lost counters and the count of received messages.

input parameter : FIFO handle returned by CAN_SetWMes

- Message opération

PROTOTYPE:	DWORD CAN_GetMessage(HANDLE,unsigned char *p);

PURPOSE:	This function read a message from the FIFO.

input parameter : 	HANDLE :FIFO handle returned by CAN_SetWMes

*p : pointer to 16 bytes length buffer for storing the incoming message.

output parameter: Returns the number of remaining messages in the FIFO plus one or CANERROR. If 0 is returned the en the FIFO is empty. .

PROTOTYPE:	DWORD CAN_SendMessage( usigned char *PTRDATA);

PURPOSE: 	This function sends a GAN message . the function doesn't wait for the message to be transmitted.

The message is copied in one of the free 82527 channel (if one is available)

It is transmitted according to the GAN arbitration rules.

If no channel is available the function return CANBUSY. The user application has to retry or eventually report time-out.

input parameters: PTRDATA: pointer to the message.

PROTOTYPE: 	DWORD CAN_RequestRemoteFrame(HANDLE hw, unsigned char *p) ;

PURPOSE: Allows to send CAN " REQUEST _REMOTE_FRAME " frame type.

input parameter : 	hw : handle returned by CAN_SetWMes

*p : Pointer to the message initialised with the requested identifier .

output parameter: Returns OK or CANERROR.

Declaration Example :

/* Request frame ident*/

unsigned char RRF[]={0,IdentH,IdentL,0,0,0,0,0,0,0,0,0,0,0,0,0};

/* Initialise Channel10*/

unsigned char Can_Config[]={10,RECEIVE,STANDARD_IDENT, 0, 0,0,0,END_TABLE};

/*Channel 10 must be configured in RECEIVE mode, using CAN_SetObjectconfig*/

CAN_SetObjectConfig(Can_Config);

/* send Request */

if (CAN_QueryRTRStatus (Myhandle) & 0x10) //checks if channel is free

CAN_RequestRemoteFrame(Myhandle , RRF);

- Status Operation

PROTOTYPE: 	DWORD CAN_GetStatus(VOID);

PURPOSE: 	This routine gets the 82527 status

output parameter: status in 82527 status register.

# Bit 7 BUSOFF

one: there is an ab normal rate of occurrences of errors on the CAN bus. This condition occurs when an error counter in the 82527 has reached the limit of 256. During busoff, no messages can be received or transmitted. The only way to exit this state is by resetting the Init bit in the Control register (location 0)

When a Busoff error occurs the driver posts a MES+2 message to the application and a recovery sequence starts on the CAN bus. See CAN_SetWMes function.

# Bit 6 WARN

one: there in an ab normal rate of occurrences of errors on CAN bus. This condition occurs when an error counter in the 82527 has reached the limit of 96 ,

When a WARN error occurs the driver posts a MES+ 1 message to the application and a recovery sequence starts on the CAN bus. See CAN_SetWMes function.

# Bit 5 Wake statt up

One: this bit is set when the 82527 had been previously set into sleep mode by the CPU. The intcan library does'nt handle the sleep mode.

# Bit 4 RxOK

One: Receive message successfully. Since this bit was last reset by the CPU, a message has been successfully received.

# Bit 3 TxOK

One: Transmit message successfully. Since this bit was last reset by the CPU, a message has been successfully transmitted.

# Bit 0-2 Last Error Code

This field contains a code which indicates the type of the first error to occur in a trame on the CAN

bus. If a message is without error the field will be cleared to 0.

0 (000) No Error .

1 (001) Stuff Error. More than 5 bits in a sequence have occurred in a part of a received message where this is not allowed.

2 (010) Form Error. The fixed format part of a received trame has the wrong format

3 (011)Acknowledgement Error. The message transmitted by this device was not acknowledged by another node.

4 (100) Bit 1 Error. During the transmission of a message, the 82527 wanted to send a recessive level (logical1 ), but the monitored CAN bus was dominant.

5 (101) Bit 0 Error. During the transmission of a message, the 82527 wanted to send a dominant level (logical 0), but the monitored CAN bus was recessive.

6 (110) CRC Error. The CRC checksum was incorrect in the message received.

7 (111) Unused.

PROTOTYPE:	DWORD CAN_QueryRTRStatus(HANDLE);

PURPOSE: 	Test if a "REOUEST _REMOTE_FRAME " request is pending.

input parameter : FIFO handle returned by CAN_SetWMes .

output parameter: Returns STATUS or CANERROR.

Bit 3-0 : unused

Bit 5-4 : TxRqst : 	10 : A REQUEST_REMOTE_FRAME is sent to request a remote node to send the corresponding data.

01 : A REQUEST _REMOTE_FRAME' has been successfully transmitted. The channel is free to accept a new request.

Bit 31-6: unused

see the CAN RequestRemoteFrame function.

PROTOTYPE:	DWORD CAN_GetDLLvers(VOID);

PURPOSE: 	Get the CANAPI.DLL version number and the kernel driver version number.

output parameter: Returns an hexadecimal value .

Low bytes: Driver version number.

High bytes: DLL version number.

Example : 	Driver version: 1.00

DLL version: 1 .02

CAN_GetDLLvers returns 0x01020100

- Direct Access

PROTOTYPE:	UNSIGNED CHAR CAN_ReadReg(UNSIGNED CHAR ad);

PURPOSE: 	The function Read from the 82527 mounted onto the PC4C board.

input parameter : 	ad: 	Address register to read.

output parameter: Value in 82527 register.

PROTOTYPE:	VOID CAN_WriteReg(UNSIGNED CHAR ad, UNSIGNED CHAR data);

PURPOSE: 	The function write to the 82527 mounted onto the PC4C board.

input parameter : 	ad: 	Register Address to write.

data: 	Data to write.

- Console Application (Windows NT only)

PROTOTYPE:	HANDLE CAN_SetProc( VOID ( *USERPROC )( UNSIGNED INT MES) );

WARNING : Windows NT only.

PURPOSE: 	The function creates a FIFO buffer dedicated to the application and registers a user call-back address which will be called when at least one incoming message becomes available in the application FIFO or when a CAN error occurs.

input parameter : 	USERPROG : 	User callback address.

MES: 			Value of the message status.

CANAPI posts MES message to the USERPROG function.

- CANAPI posts " 1 " when incoming CAN messages are available the user's application must read ali the available messages to trig the post function again.

- CANAPI posts " 2 " when " CAN warn error " condition occurs on GAN bus

- CANAPI posts " 4 " when " CAN Bus off error " condition occurs on GAN bus

output parameter: Handle to FIFO or CANERROR .